Supplementary Material

# Sample collection

Aerial microbiome samplers (typically termed bioaerosol samplers) can be categorised into several main functional types (Figure S1, see Ghosh et al., 2015; Haig et al., 2016; and Lindsley et al., 2017 for comprehensive overviews). Within each functional type, samplers tend to have a set of common advantages and limitations (see Table S1). For example, filters tend to have high collection efficiencies, but poor potential for the maintenance of biological viability. However, even within categories, the available samplers are highly diverse and can have variable sampling capacities and characteristics (such as flow rate, collection efficiency, and collection medium, e.g. see Haig et al., 2016; Kesavan and Sagripanti, 2015; Wang et al., 2015 for a summary). A comprehensive list of bioaerosol samplers, together with their particle size range and application suitability is provided in Lindsley et al. 2017.

Figure 1. Schematic illustrating the key types of bioaerosol samplers: (a) Filter (IOM sampler); (b) Cascade impactor (Sioutas) (c) Cyclone (aluminium cyclone); (d) Impinger (glass midget impinger mounted on pump). Images courtesy of SKC Inc.

The key implication of the diversity and differences between samplers is that sampler choice strongly affects the outcomes of aerial microbiome studies. Results from studies using different samplers are not easily comparable, and in some cases inappropriate sampler choice may make a study’s conclusions invalid. For example, Kesavan and Sagripanti (2015) show that inadequate sampler selection can result in a serious underestimate of infectious disease risk, the same principles would apply when sampling UAMs and could lead to an underestimation of ecosystem service potential. Although sampling technologies are continuing to advance and evolve, standardisation remains unlikely given that sampler choice will remain guided by study design and questions. However, the obstacle of different sample collection techniques for understanding UAM’s for ecosystem service provision will be minimized if techniques for downstream genetic and statistical analysis are unified and data sharing implemented. Here we describe some of the benefits and drawbacks of the various sampling methods.

# Table S1. Core categories of microbiome (bioaerosol) samplers and their key advantages and limitations. Based on comprehensive reviews in Ghosh et al., 2015; Haig et al., 2016; Kesavan and Sagripanti, 2015; Lindsley et al., 2017. Note that within each category there are numerous individual sampler types, which may have widely varying specific sampling characteristics and will be of varying suitable for different bioaerosol types. For a comprehensive list of specific samplers, their particle size range and application suitability (e.g. culture/microscope/immunoassay/genomic studies), see Lindsley et al., 2017.

| Sampler type | Advantages | Disadvantages | Key sub-types | Common examples |
| --- | --- | --- | --- | --- |
| FiltersPFB | Flexible application - many types available Cheap and simple to deploy Many samplers are lightweight/portable → suitable for personal monitoring Some sample inhalable fraction (e.g. Button) | Subject to collection/viability loss via desiccation, deposition on sampler walls, incomplete elution from filter, filter overloading Requirement for pumps & power → limitations on field deployment Must be preceded by a size selective inlet (e.g. cyclone/ impactor) for size classification | Cellulose filter Gelatin filter Glass fibre filter PTFE filter | IOM Button PAS-6 |
| ImpactorsPFB | Flexible application - many types, varied flow rates & collection media (agar, glass slides, liquid) Direct-collection onto growth/microscopy media→ reduced post-processing Some offer size classification, inhalable fraction sampling and/or long-term sampling capacity | Collection media (plates/slides) can become overloaded making enumeration difficult Subject to collection/viability losses via shear forces, desiccation, particle bounce, re-entrainment, inlet losses, deposition build up | Slit impactor Cascade impactor Virtual impactor | Hirst-type Rotorod BioStage Impactor Anderson multi-stage |
| CyclonesFBV | Minimises desiccation and shear/impaction stress → suitable where viability is important Less prone to particle bounce than impactors → good collection efficiency Multi-stage versions → size classification | Collection efficiency curves less sharp than most impactors Subject to collection/viability losses via shear forces, liquid carry-over, evaporation, adherence to cyclone walls | Wet cyclone Dry cyclone | NIOSH one stage NIOSH two -stage Coriolis PAS-5 |
| ImpingersFBV | Minimises desiccation → suitable where viability is important Widely used → good info. on collection efficiency | Collection/viability losses due to shear forces, re-aerosolization, evaporation, wall adherence Requires post-processing for analysis Evaporation limits collection time Relatively expensive and fragile | Single-stage Multi-stage | Burkard Multi-stage AGI-30 Impinger Midget Impinger |
| ElectrokineticPFBV | Cheap and simple to deploy Much higher collection efficiency than passive samplers Low desiccation/impaction stress Lack of pump/low power requirement → flexible field deployment, can be unattended long-term | Lower collection efficiency than most pump-based samplers (e.g. filters) Electrical charge may affect bacterial viability Limited studies to date → relative lack of information on sampling performance | n/a | Rutgers Electrostatic Passive Sampler Inspirotek Sampler |
| CondensationPFBV | Can collect ultrafine bioaerosol particles Maintains microorganism viability | Complex, multi-component and expensive | n/a | LSS100 |
| Real-timePFB | Capable of real-time and large-scale surveillance Strong potential for greater technological development | Lower precision than other methods, possible interference from non-biological material ID affected by fragmentation & orientation Limited studies to date → relative lack of information on sampling efficiency, precision | Fluorescence & light scattering Flow cytometry | WIBS Plair2000 |
| PassivePFB | Extremely cheap and simple to deploy Efficient means of obtaining preliminary/qualitative information Suitable for culturing without post-processing Does not disturb airflow Reproduces real conditions | Volume of air unknown → cannot quantify micro-organism concentrations, not comparable with active sampling methods Reliance on settling → collection bias towards larger particles Generally not suitable for long term continuous sampling (e.g. agar dries out after 4 hrs) | Agar settling plates PTFE filter settling plates Electret cloth |  |

- 1P=Pollen, F=Fungi, B=Bacteria/Archaea, V=Viruses

Works Cited

Ghosh, B., Lal, H., Srivastava, A., 2015. Review of bioaerosols in indoor environment with special reference to sampling, analysis and control mechanisms. Environ. Int. 85, 254–272. https://doi.org/10.1016/j.envint.2015.09.018

Haig, C.W., Mackay, W.G., Walker, J.T., Williams, C., 2016. Bioaerosol sampling: Sampling mechanisms, bioefficiency and field studies. J. Hosp. Infect. 93, 242–255. https://doi.org/10.1016/j.jhin.2016.03.017

Kesavan, J., Sagripanti, J.L., 2015. Evaluation criteria for bioaerosol samplers. Environ. Sci. Process. Impacts 17, 638–645. https://doi.org/10.1039/c4em00510d

Lindsley, W.G., Green, B.J., Blachere, F.M., Martin, S.B., Law, B.F., Jensen, P.A., Schafer, M.P., 2017. Sampling and Characterization of Bioaerosols, in: Ashley, K., O’Connor, P.F. (Eds.), NIOSH Manual of Analytical Methods. National Institute for Occupational Health and Safety, Washington DC, pp. 82–112. https://doi.org/10.1103/PhysRevB.71.165307

Wang, C.H., Chen, B.T., Han, B.C., Liu, A.C.Y., Hung, P.C., Chen, C.Y., Chao, H.J., 2015. Field evaluation of personal sampling methods for multiple bioaerosols. PLoS One 10, 1–19. https://doi.org/10.1371/journal.pone.0120308